Controller Install introduction

Controller support two working mode- With driver & without driver. The one with driver can be worked under OS WinXP/Win7 32bits/Win7 64bitsWin CE/WinXPE. The one Without driver can be worked under OS Windows, Linux, Android .

With driver and without driver switch: controller has been set as without-driver controller when it was produced , client need to install the driver program from the Driver file in the CD. According to the corresponding operating system to install the suitable driver.

If you need to use the no-driver type,please click Cali-PNP calibration program in PNP file(Please click the right mouse underWin7,then to operate calibration program as administrator),and follow the below setting :

PNP：Choose this section, click “Save set” to set the controller to without driver. Note: only select this function ,the other function setting can take effect .

Active Time：Default value is 0, if the client need Right-click function ,just select the time of the appearance of right-click,click “Save set” is OK .

Double Click Setup：

Height：The Double-click area’ height of Mouse and Touch screen.

Width：The Double-click area ‘ width of Mouse and Touch screen.

This function main set the double-click area,It usually has been set the default values in the factory ,the client need not to adjust.

Setp：Default is 12, if the Double-click is not sensitive ,you can increase the step length, range from 6 to 28.

Press：Touch force,the factory default is 10,normally need not to adjust. touch force is divided into 28 levels, you can set the touch screen sensitive via changing numerical value. If the customer need better handle,please turn down the numerical value,instead, turning up the numerical value to increase the local anti-jam capabilities. Then click “Save set”.

Save Set：Save the Settings in the firmware, and take effect immediately.

Draw Test：Customers can do click and draw lines testing in the drawing board

UnTouch：Touch has no date output after selecting .

Touch：Touch can normal output data after selecting.

Calibrate：Four point calibration ,calibration screen will pop up after clicking, follow the screen prompt to finish it. After install no-driver program, usually need to calibrate the cursor to ensure normal touch .